- Suspended ceiling description

- Range Lauder LINEA

- Product reference SWELL

Patented system

Suspended ceiling lot

## General

The suspended ceiling will be created using SWELL panels product reference from the Lauder LINEA range of the Laudescher brand, which are compliant with standard EN 13964 Suspended ceilings.

The suspended ceiling will be installed in compliance with the manufacturer's recommendations

and installation good practice rules.

The service will include the supply and installation of the panels, with or without acoustic materials,

the primary grid system (if necessary), hanging rods and all related services such as: erection

of scaffolding, cutting operations, blockouts, offcuts and reveals, in accordance with the assembly drawings provided by the company.

## Description of the suspended ceiling

## The suspended ceiling will be of the following type:

Range: Lauder LINEA

Product reference: SWELL

Form: Linear wave

Total thickness: 68 mm

Slat section: 20 x 68 mm

Facing side: 20 mm

Slat thickness: 68 mm

Rear black wooden counter-slats: 20 x 42 mm

Spacing between slats: 100 mm

Average void area: 83%

Reaction to fire: B-s1,d0 / B-s2,d0 as defined in standard EN 13501-1

Solid wood:

- Pine

- Douglas Fir

- Oak

Finishing: (in accordance with building class)

- Natural

- Clear varnish

- Wax Color

- Wax Color MC

Wax Color or Wax Color MC finishing on Pine, with the following choice available to contractors: Honey, Oak, White, Wenge, Cherry, Chocolate, Mahogany, Grey.

- Edging system: edge A

- Modular dimensions: 1700 x 1200 mm

## Fire recommendations

The product must be supplied with a test report performed on the finished product, in acoustic or open version, in an approved laboratory in accordance with standard EN 13501-1.

Reaction to fire: B-s1,d0 / B-s2,d0 as defined in standard EN 13501-1

- Technical performance

- The product must have a VOC classification in accordance with EN ISO 16000-3, 6, 9 and 11

in compliance with current legislation

- PEFC / FSC: the product must be supplied with a certificate and individual number

- Humidity resistance: in accordance with building class

- Maintenance: Keep free of dust. Lauder LINEA panels must not be cleaned with detergents or water-based products under any circumstances

- Storage: Products in the Lauder LINEA range must be stored flat, in premises with constant indoor humidity sheltered from water and blown air heating. Products in the Lauder LINEA range must be stabilised in the destination premises for at least 48 hrs prior to installation.

- Installation

- The ceiling is hung on a minimum of two Ø6 mm threaded rods per panel, compliant with current standards and good practice rules in each country.

The system gives the panels a perfectly flowing and seamless appearance.